Effects of gabion steps on hydraulic jump characteristics downstream of stepped spillways

This study explored the hydraulic jump characteristics and rate of energy dissipation downstream of gabion-stepped spillways under different flow regimes. A stepped spillway contains a set of steps beginning from its crest and continuing to its downstream chute to artificially roughen the spillway surface and thereby increase energy dissipation. Gabion steps offer a beneficial alternative to solid, impermeable steps owing to their economic advantages, easy implementation, stable water pressure reduction, and excellent energy dissipation. As the step configuration is critical to realizing an optimal hydraulic design, six stepped spillway models were evaluated in this study on a 26.6° slope (a 1:2 vertical:horizontal ratio), representing one solid-stepped spillway and five gabion-stepped spillways with different arrangements. The results indicated that in the nappe flow regime, seepage flow through the pores of the gabion aggregate and overflow passing the gabion steps increased the energy dissipation associated with a smaller sequent depth ratio, relative jump length, and roller length compared to solid, impermeable steps. The reduction in the hydraulic jump length depended on the flow regime and the gabion step arrangement. In the skimming flow regime, smaller energy dissipation rates were measured downstream of the gabion steps, and when gabion was used for all steps, the sequent depth ratio, jump length, and roller length decreased by an average of 3.34%, 8.61%, and 8.14%, respectively, compared to solid steps. The maximum energy dissipation measured at the downstream end of a gabion-stepped spillway was 74.4%, which was 5.03% greater than that for the solid-stepped spillway. Finally, the average dimensionless residual head for the solid-stepped spillway was 4.65 compared to 4.21 for the gabion-stepped spillway under the same flow conditions.

Introduction

Stepped spillways contain a set of steps that begin at the crest of the spillway and continue to its downstream chute. These steps artificially roughen the spillway bed to dissipate more energy than other spillway types. Consequently, considerable attention has been paid to this type of spillway in recent years (Rajaratnam, 1990; Rice and Kadavy, 1996; Chanson and Toombes, 2002; Gonzalez and Chanson, 2004; Felder and H, 2014; Ghaderi and Abbasi, 2021). Various studies have investigated the influence of the chute slope and number of steps, evaluated the scale effect, and determined the effects of step geometry on the flow characteristics over stepped spillways (Mero and Mitchell, 2017; Al Talib, Mohammed, and Hayawi, 2019; Felder, Geuzaine, Dewals, and Erpicum, 2019; Ghaderi, Abbasi, Abraham, and Azamathulla, 2020; Ghaderi, Abbasi, and Di Francesco, 2021).

The use of gabion steps, which are rectangular baskets (often made of a galvanized steel mesh) filled with rocks of various sizes (Stephenson, 1979), provides a simple and cost-effective method for constructing stepped spillways (see Figure 1).

Gabion steps may be suitable for low-to-medium head stepped spillways. Furthermore, gabions are stable, low cost, flexible, and porous (Chanson, 2001). Notably, the energy dissipation of the flow over a gabion-stepped spillway is considerable owing to seepage flow through the porous bodies of the gabion steps, thereby reducing the need for a downstream stilling structure.

Source: The effect of gabion steps on the hydraulic jump characteristics downstream of stepped spillways by Rasoul Daneshfaraz, Hojjat Sadeghi, Amir Ghaderi & John Patrick Abraham used under CC-BY with modifications