Effects of Gabion Steps on Hydraulic Jump Characteristics Downstream of Stepped Spillways

Ashley M. Author, Ph.D.,1 Jackson Q. Writer,2 and Charlie O. Researcher, P.E., M.ASCE3

1Head Engineer, Department of Civil and Environmental Engineering, Research State University, City, ST, 99999-1111; email: amauthor@rsu.edu (Corresponding author)

2Researcher, Department of Civil and Environmental Engineering, Research State University, City, ST, 99999-1111; email: jqwriter@rsu.edu

3Lead Scientist, Department of Civil and Environmental Engineering, International Research University, City, Country, 99999-1111; email: coresearcher@iru.edu

ABSTRACT:

Gabion steps, which comprise rock-filled rectangular baskets, offer a beneficial alternative to solid, impermeable steps in spillways owing to their economic advantages, easy implementation, stable water pressure reduction, and excellent energy dissipation. This study explored the hydraulic jump characteristics and rate of energy dissipation downstream of gabion-stepped spillways under different flow regimes. As the step configuration is critical to realizing an optimal hydraulic design, six stepped spillway models were evaluated in this study on a 26.6° slope (a 1:2 vertical:horizontal ratio), representing one solid-stepped spillway and five gabion-stepped spillways with different arrangements. The results indicated that in the nappe flow regime, seepage flow through the pores of the gabion aggregate and overflow passing the gabion steps increased the energy dissipation associated with a smaller sequent depth ratio, relative jump length, and roller length compared to solid, impermeable steps. In the skimming flow regime, smaller energy dissipation rates were measured downstream of the gabion steps, and when gabion was used for all steps, the sequent depth ratio, jump length, and roller length decreased by an average of 3.34 %, 8.61%, and 8.14%, respectively, compared to solid steps. The reduction in the hydraulic jump length depended on the flow regime and the gabion step arrangement. The maximum energy dissipation measured at the downstream end of a gabion-stepped spillway was 74.4%, which was 5.03% greater than that for the solid-stepped spillway. Finally, the average dimensionless residual head for the solid-stepped spillway was 4.65 compared to 4.21 for the gabion-stepped spillway under the same flow conditions.

PRACTICAL APPLICATIONS (Optional):

AUTHOR KEYWORDS (OPTIONAL): Gabion step, hydraulic jump, stepped spillway, nappe flow, skimming flow

INTRODUCTION:

Stepped spillways contain a set of steps that begin at the crest of the spillway and continue to its downstream chute that artificially roughen the spillway bed to dissipate more energy than other spillway types. Consequently, considerable attention has been paid to this type of spillway in recent years (Rajaratnam 1990, Rice and Kadavy 1996, Chanson and Toombes 2002, Gonzalez and Chanson 2004, Felder and H 2014, Ghaderi and Abbasi 2021). Various studies have investigated the influence of the chute slope and number of steps, evaluated the scale effect, and determined the effects of step geometry on the flow characteristics over stepped spillways (Mero and Mitchell 2017, Al Talib et al. 2019, Felder et al. 2019, Ghaderi et al. 2020, Ghaderi et al. 2021).

The use of gabion steps, which are rectangular baskets (often made of a galvanized steel mesh) filled with rocks of various sizes (Stephenson 1979), provides a simple and cost-effective method for constructing stepped spillways (see Figure 1).

Furthermore, gabions are stable, low cost, flexible, and porous (Chanson 2001). Notably, the energy dissipation of the flow over a gabion-stepped spillway is considerable owing to seepage flow through the porous bodies of the gabion steps, thereby reducing the need for a downstream stilling structure. Gabion steps may be particularly suitable for low-to-medium head stepped spillways.

MAIN TEXT:

CONCLUSIONS:

DATA AVAILABILITY STATEMENT:

ACKNOWLEDGEMENTS (Optional):

NOTATION LIST (Optional):

REFERENCES:

TABLES:

FIGURE CAPTION LIST:

FIGURE FILES:

Source: The effect of gabion steps on the hydraulic jump characteristics downstream of stepped spillways by Rasoul Daneshfaraz, Hojjat Sadeghi, Amir Ghaderi & John Patrick Abraham used under CC-BY with modifications